| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Cannulation of liquids under inert atmosphere into Aldrich Sure-Seal bottles |  |  |
| Applies to Lab (name): Chamberland Group | SCI Room #: 313 |  |
| Prepared By: Jennifer Reeves, Stephen Chamberland | Date: 5/30/12 | Revision: 8/20/12 |

# Special Notifications:

# Hazard Description:

- Glassware under pressure

- Potentially flammable liquids

- Sharp needle

- Compressed gasses

- Open flame

# Protective Equipment:

- Fume hood with sash

- Lab coat

- Protective wrap-around goggles

- Nitrile gloves

- Heat-resistant cloth gloves

# Engineering Controls:

Perform procedure in a working fume hood.  Remove all spark and ignition sources, as well as all open flames, if working with a flammable solvent.

# Special Handling or Storage Requirements:

Refer to SOP on "Capping Aldrich Sure-Seal bottles with the crown cap crimper" for storage of the liquid in the Sure-Seal bottle when the cannulation is complete.

# Spill Response Procedure:

Close fume hood and let solvent evaporate inside of fume hood.

# Decontamination/Waste Disposal:

Dispose of any organic solvent waste in the halogenated organic solvent waste container.

# Special Emergency Procedures:

If a toxic chemical comes into contact with nitrile gloves, remove the gloves as soon as possible and set them in the fume hood.

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

# Step by Step Procedure:

1) Steps 1-3 must be done quickly.  Obtain a clean 125 or 500 mL amber storage bottle filled with about 1/2 inch of activated 4 angstrom molecular sieves that has been oven-dried overnight at 130 degrees C or flame dried (by Dr. Chamberland, see step 4) and cooled under a positive pressure of inert gas.  This amber storage bottle will be used to store the reagent/solvent.

2) Place a septum upside down on top of the warm amber storage bottle.

3) The air inside the warm amber stoarge bottle will need to be flushed out as follows:

a) Take the needle found on the manifold and puncture it through the septum.

b) Open up the nitrogen gas so that it begins flowing through the needle into the amber storage bottle.

c)  Insert an unobstructed exit 16-gauge or 20-gauge disposable needle into the septum to allow the air to be

flushed out of the amber storage bottle.

d) Close off the bubbler by turning the three way stop cock to the left. This will allow all of the nitrogen pressure to

go through the amber storage bottle to flush out any remaining air.  Check to see that nitrogen is flowing through

the exit needle.

4) Have Dr. Chamberland flame dry the amber storage bottle under inert gas.  Once the flame-dried amber storage bottle has cooled to ambient temperature, wrap parafilm around the outside of the septum to seal the septum to the glass neck of the amber storage bottle.

5) If the liquid has been recently distilled (see SOP "Distillation of Solvent with a Reactive Drying Agent Under Inert Atmosphere or Vacuum"), it should also be under inert gas in a round-bottomed flask sealed with a rubber septum.

6) Secure the amber storage bottle and the flask containing the freshly distilled liquid to the metal bars using a three-prong clamp. At this point, the amber storage bottle should have an inert gas line coming into it as well as an exit needle attached.  The flask containing the freshly distilled liquid should also have an inert gas line coming into it.  The bubbler should be closed.

7) Transfer the liquid distilled into the receiver flask into the amber storage bottle:

a) Simultaneously insert both ends of the metal cannula through the septum of both containers.  The blunt end of

the cannula should be placed in the round-bottomed flask containing the freshly distilled solvent.  Remove the

inert gas line from the amber storage bottle.  Check to see that nitrogen is flowing through the exit needle in the

amber storage bottle.

b) Lower the blunt end of the cannula down into the freshly distilled liquid in the round-bottomed flask. The positive

pressure of inert gas will transfer the liquid from the round-bottomed flask through the cannula and into the

amber storage bottle.  If the liquid does not transfer after a minute or so, raise the round-bottomed flask above

the level of the amber storage bottle.  If that doesn't work, increase the inert gas pressure slightly.  Do not

increase the pressure too high.

8) Once the transfer is complete, remove the disposable exit needle from the storage bottle and open the bubbler back up.

9) Transfer the inert gas line from the round-bottomed flask into the amber storage bottle.

10) Remove the cannula from both containers (starting with the amber storage bottle first).  The amber storage bottle should still have the inert gas line and the round-bottomed flask should have nothing in the septum.

11) Remove the flask and clean it properly.  Dispose of any residual solvent in the organic solvent waste.  If molecular sieves were used, they can most likely be left to dry and reactivated later.

12) Remove the inert gas line from the amber storage bottle.

13) Turn off the nitrogen gas.

14) Cap the storage bottle (refer to SOP “Capping Sure-Seal bottles with the crown-cap crimper”).

Approved By (PI) : Stephen Chamberland, Ph.D.					Date: 8/20/12

PI Signature: __________________________________________________

EH&S Signature: ________________________________________________		Date: ______________