| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Capping Aldrich Sure-Seal bottles with the crown cap crimper |  |  |
| Applies to Lab (name): Chamberland Research Group | SCI Room #: 313 |  |
| Prepared By: James Siegenthaler, Jennifer Reeves, Stephen Chamberland | Date: 5/8/12 | Revision: 8/29/12 |

# Special Notifications:

This procedure enables chemical storage in an air- and moisture-free environment.  It is usually done after a freshly distilled liquid is transferred into a clean Aldrich Sure-Seal amber storage bottle or to replace a broken or worn Aldrich Sure-Seal crown cap assembly on an opened reagent bottle.

# Hazard Description:

Broken glass when excessive pressure is used. If the bottle breaks, then hazardous chemicals can spill. Liquid stored in the bottle can leach out if the Sure-Seal elastomer barrier or metal crown cap has degraded.  A razor blade will be used to cut plastic at the end of the procedure, so be sure that the razor blade is never directed toward any part of the body while cutting.

# Protective Equipment:

- Chemically resistant standard nitrile gloves or heavy-duty nitrile gloves if necessary

- Protective wrap-around goggles

- Lab coat

# Engineering Controls:

Perform the procedure on a clean and uncluttered lab bench.  Ensure glassware is free of cracks.  Remove all ignition sources and open flames from the immediate work area.  If utilizing extremely hazardous or corrosive chemicals, then perform the procedure behind a working hood sash in a well-ventilated fume hood.  When working with air- and/or moisture-sensitive substances, this process should be performed quickly.

# Special Handling or Storage Requirements:

C

The equipment is heavy, and the glass jars are fragile. Care is to be given during usage and transport to prevent damage.

# Spill Response Procedure:

Consult the SDS sheet of the chemical spilled for the proper clean up procedure.

# Decontamination/Waste Disposal:

A

Consult the SDS sheet for the chemical contained and proper waste and storage procedures.

# Special Emergency Procedures:

B

If any part of the hand is cut, apply antibiotic ointment and a bandage to the wound if needed.

D

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

E

Step by Step Procedure:

- On a 2x4” self-sticking label produced by the group, write the name and/or formula of the chemical to be stored in the amber storage bottle.

- Affix the label to the bottle.

- Using the allen wrench provided, adjust the height of assembly A on the back post (D) so that the bell (B) that secures the crown is about ¼” above the top of an empty bottle that is the same size as the bottle to be capped.  Leave room for the bell (B) to easily descend onto the bottle top and enable the crown cap to be crimped. To loosen this assembly, use the allen wrench on the back of the metal block in part A.  When it is loose, slide assembly A up or down on the support stand D until it reaches the correct height.

- Transfer the chemical to be stored to a clean, dry Sure-Seal amber storage bottle (see the SOP “Cannulation of liquids under inert atmosphere into Aldrich Sure-Seal bottles”). If the chemical to be stored is a solvent that has just been distilled under inert atmosphere or vacuum, the bottle will likely have a rubber septum on the top and be sealed with parafilm. Because removing this septum will expose the chemical to the atmosphere (and, thus, to air and moisture), it is important to execute steps 5-9 as quickly as possible. Read and understand steps 5-9 carefully before proceeding.

- Remove the parafilm and septum, then quickly place a PTFE-faced plastic wafer on the top of the bottle with the white side up.

- On top of the PTFE-faced plastic wafer, place an uncrimped metal crown cap.

- Place the amber storage bottle with the crown cap and wafer resting on top of the bottle onto metal surface E, under the metal crimping bell (B).

- Gently lower the arm (D) to lower the metal crimping bell (B) onto the bottle. Make sure the bottle is properly aligned before any pressure is placed on the crown cap.

- Once the bottle is properly aligned and the bell (B) is touching the top of the metal crown cap, gently but firmly press down the lever (D) until the crimping bell (B) goes past the initial resistance and then stops.  That resistance is when the crimping of the metal crown cap takes place.  Do not use more force than is needed to go past that stopping point because the bottle can shatter.

- Using one hand, firmly hold the bottle against the metal stand E, slowly raise the lever (D) with the other hand until the bottle cap disengages from the crimping bell (B).  Ensure that the bottle does not rise off of the metal stand and then crash back down as it is being held.

- Using a metal spatula, razor blade, or a pair of pliers, remove the thin strip of plastic from the red bottle cap.  Screw on the red plastic bottle cap. Wrap Parafilm around the bottom edge of the bottle cap to seal the cap to the bottle. Store the bottle in accordance with its contents.

Approved By (PI) : Stephen Chamberland, Ph.D.					Date: 8/29/12

PI Signature: __________________________________________________ 		Date: ______________

EH&S Signature: ________________________________________________		Date: ______________