| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Changing the desiccant for the desiccator |  |  |
| Applies to Lab (name): Chamberland Research Group | SCI Room #: 313 |  |
| Prepared By: Stephen Chamberland | Date: 8/31/12 | Revision: 8/31/12 |

# Special Notifications:

This procedure describes how to change the calcium sulfate desiccant in the desiccator to maintain the low humidity environment of the desiccator. The desiccant in the desiccator needs to be changed when the indicating drierite that is sprinkled on top of the bed of white calcium sulfate in the desiccator pan changes from blue (indicating that it is dry) to a purple or pink (indicating that it is wet) color.

# Hazard Description:

The beaker of dried calcium sulfate in the oven is hot and requires thermally insulated gloves during handling. When pouring out the beaker of dried calcium sulfate from the oven into the desiccator pan, dust is produced, so hold your breath during this step.

# Protective Equipment:

- Lab coat

- Protective wrap-around goggles

- Long pants

- Close-toed shoes

- Thermally insulated gloves

# Engineering Controls:

N/A

# Special Handling or Storage Requirements:

N/A

# Spill Response Procedure:

Spilled calcium sulfate can be swept up with the dust pan and hand broom and then disposed of in the trash.

# Decontamination/Waste Disposal:

Spilled calcium sulfate can be swept up with the dust pan and hand broom and then disposed of in the trash.

# Special Emergency Procedures:

N/A

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

# Step by Step Procedure:

- To change the desiccant, remove the desiccant tray from the desiccator and pour the desiccant into an empty 1L glass beaker.  One’s hands may be used (while also holding the plastic tray) to funnel the desiccant into the beaker to minimize spillage of the desiccant.

- Once the desiccant has been emptied into the new beaker, place this beaker into the oven to dry it.

- Remove the (hot) beaker full of dried calcium sulfate from the drying oven and pour it into the plastic desiccator tray.  This step will produce dust, so hold your breath until the beaker has been poured out and while spreading out the desiccant.

- Flatten the mound of desiccant so that it is even with the lip of the plastic tray.

- Sprinkle a few pieces of blue (expensive) Drierite (calcium sulfate) on top of the non-indicating (white) Drierite.

- Replace the plastic tray in the desiccator to allow the desiccant to cool in the dry atmosphere of the desiccator.

- If needed, apply vacuum grease to the rubber seal of the desiccator to maintain a good seal when closed.

Approved By (PI) : Stephen Chamberland, Ph.D.					Date: 8/31/12

PI Signature: ___________________________________________________		Date: ______________

EH&S Signature: ________________________________________________		Date: ______________