| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Flame Drying Glassware Using the Double Manifold and Positive Inert Gas Pressure |  |  |
| Applies to Lab (name): Chamberland Group | SCI Room #: 313 |  |
| Prepared By: Jacob Buchanan and Stephen Chamberland | Date: 11/30/11 | Revision: 8/28/12 |

# Special Notifications:

For PI use only.  This procedure may only be done by Dr. Chamberland.  Students may not do this procedure.

# Hazard Description:

-Open Flame

-Sharp Needle

-Compressed Gases

-Hot Glassware

-Glassware Under Pressure

# Protective Equipment:

-Protective wrap-around goggles

-Lab coat

-Heat-resistant gloves

# Engineering Controls:

Perform procedure in a working fume hood

# Special Handling or Storage Requirements:

N/A

# Spill Response Procedure:

N/A

# Decontamination/Waste Disposal:

N/A

# Special Emergency Procedures:

Treat any minor skin burns caused by contact with the propane torch, hot glassware, or burning rubber, cork, or cloth by washing under cool water, application of burn ointment, antibiotic ointment, and a bandage if necessary.

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

# Step by Step Procedure:

1) Prepare the clean glassware that is to be flame dried by placing a dry magnetic stir bar in the flask (if necessary for a reaction or distillation).  Dry the flask above 130 degrees C in an oven overnight.  Once out of the oven, seal the flask with a rubber septum.  Completely fold the rubber septum over the neck of the flask.  If removing hot glassware from the oven, handle the glassware with hot gloves before attempting to attach the septum, and complete steps 4-8 while the glassware is hot.

2) Open the main valve (Figure 1, C) on the inert gas tank that serves the fume hood and double manifold to be used.

3) Slowly open valve A to introduce gas flow to the double manifold.  Bubbles emanating from the bubbler near the manifold indicate that a positive pressure of inert gas is present.  For flame drying a small (less than 250mL) flask, a high flow rate of about 3-5 bubbles per second is desired.  A higher bubbling rate (5-8 per second) is desired when flame drying larger flasks.

4) Puncture the septum with an unobstructed needle attached to one of the Tygon tubes connected to the double manifold.

5) Turn the stopcock valve on the double manifold connected to your tubing so that the blue glass tip points up to introduce gas flow through the tubing.

6) Puncture the septum with a second unobstructed, disposable needle ("the vent needle"). Use a hole that is already present in the septum if possible.

7) Ensure that a positive pressure of inert gas is emanating from the vent needle by holding the vent needle up to a sensitive area of the skin such as the moistened underside of the wrist or the lips.

8) Turn the three-way valve connecting the manifold to the bubbler so that inert gas no longer flows to the bubbler.  Bubbling should cease.

9) Ignite the propane torch and set it in the fume hood.  Ensure that no flammable solvents or combustible materials are nearby, and that the lit torch is not pointed toward you or anyone else.

10) Pick up the piece of glassware by the septum or the needle caps and hold the bottom of the glassware at an angle in the hottest part of the torch flame.

11) Initially, moisture will appear on the inside wall of the glassware. Rotate the glassware and move it through the flame until no more moisture appears on the inside. Do not put the septum in the flame or it will melt/burn.  Use a cork ring to brace against the bottom of the glassware to aid in control of the glassware while moving it along the flame.  Do not put the cork ring in the flame or it will catch fire.

12) Once all the visible moisture appears to be gone, keep heating the flask until the gas emanating from the vent needle is warm.  One to five minutes of heating may be required depending on the glassware size.

13) Place the glassware on a cork ring in the fume hood to cool to room temperature under positive inert gas pressure.  Cooling may take between five to fifteen minutes.  The higher initial flow of inert gas prevents air from rushing into the flask as it cools because of the decrease in temperature.  If additional glassware needs to be flame dried, repeat steps 3-7 (use a different Tygon tube in step 4) and 10-13.  Then continue to step 14.

14) After placing the hot glassware on the cork ring, turn off the propane torch and store it properly.

15) When the glassware has cooled to room temperature, quickly remove the vent needle and then immediately turn the three-way valve to resume inert gas flow to the bubbler.  Failure to adjust the three way valve quickly will cause pressure to build up in the manifold.  Excessive gas pressure from a manifold is most commonly alleviated by popping the septum off of the glassware and/or forcibly ejecting one of the glass stopcocks toward the user.  This is to be avoided.  Store the vent needle in a plastic cover for future use.  If the vent needle is or becomes contaminated or obstructed, dispose of it in the sharps waste and use a new disposable needle.

16) Adjust valve A to decrease the rate of inert gas flow to one bubble every 3 to 5 seconds if continued inert gas use is needed.  If inert gas is no longer needed, close this valve to finger tightness and then close valve C to finger tightness.

17) The piece of glassware is now ready for use.

*Make sure to put the needle attached to the Tygon tubing back into the container with drying agent when no longer needed.

Approved By (PI) : Stephen Chamberland, Ph.D.					Date: 11/30/11

PI Signature: __________________________________________________

EH&S Signature: ________________________________________________		Date: ______________