| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Using the Thermocoupled Hot Plate |  |  |
| Applies to Lab (name): Chamberland Research Group | SCI Room #: 313 |  |
| Prepared By: James Siegenthaler, Stephen Chamberland | Date: 8/29/2012 | Revision: 10/7/14 |

# Special Notifications:

The surface of the hot plate and the oil bath may reach a temperature sufficient to cause burns to the skin.

# Hazard Description:

The surfce and anything located on the surface of the hot plate is hot and can burn skin that comes in contact with the hot plate surface, the oil bath, or the hot oil.  Hot oil can splash out of the bath if the bath is bumped into, causing burns if the oil is hot enough.  If the oil bath falls off the surface of the hot plate, it can crack or break, causing hot oil to spill onto the user and/or the surface of the fume hood.

# Protective Equipment:

- Lab coat

- Protective wrap-around goggles

- Chemically resistant nitrile gloves

- Hot gloves

- Fire exstinguisher

# Engineering Controls:

# The hotplate should be placed in a clean, uncluttered, and well-ventilated fume hood.  Provide a stable and immovable surface for the hot plate so that if the plate is lightly bumped, it will not tip or fall.  The hot plate has a vertical metal bar onto which flasks and/or other apparatus can be clamped.  Alternatively, the metal bars in the fume hood can be used to secure items with clamps.  Strong and sturdy metal three-prong clamps must be used to secure flasks and other items placed on the hot plate or oil bath.  The location of the hotplate should be kept dry and free from moisture, which can cause corrosion or electric shock. It is recommended that the hotplate be placed on a stable LabJack to facilitate height adjustment. The power cords should be securely attached to a wall outlet, kept out of the way and bundled to prevent tripping, and be checked frequently for signs of damage.

# Special Handling or Storage Requirements:

Store in a secure, stable, dry location. Do not store chemicals on the surface of the hot plate. When in use, ensure that it is placed in a safe location and is fully stabilized on a clean, dry workspace.

# Spill Response Procedure:

If chemicals are spilled on top of the device, turn it off, unplug it, and allow the hotplate to cool before preceding to clean. Once the device has cooled to ambient temperature, clean the exterior thoroughly using paper towels moistened with water or an organic solvent like acetone or hexane.  Do not immerse the equipment in water or any other liquid.  Avoid getting liquid in the interior of the device.

# Decontamination/Waste Disposal:

Any chemical spilled on the deivce should be cleaned up as recommended in the chemical’s SDS.  Usually, water followed by an organic solvent like acetone or hexane can be used to clean up a spill with paper towels.  Spilled silicon oil can be cleaned up using hexane and paper towels.  Ensure that chemically resistant nitrile gloves are used when cleaning up silicon oil using hexane.  After the hexane wetted paper towels are allowed to dry in the fume hood, they may be discarded in the trash if they were only used to clean up spilled silicon oil.

# Special Emergency Procedures:

If a fire should occur, unplug the hot plate, then extinguish the flames using the appropriate fire extinguisher.

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

# Step by Step Procedure:

- An uncluttered and clean fume hood is the ideal workspace for using the hot plate.  Set the hot plate on a firm, level, and unmovable surface, such as the floor of the fume hood or on a lab jack that sits on the floor of the fume hood.  If the height of the hotplate needs to be adjusted, place the hotplate on a lab jack that can be raised or lowered. Adjust the height of the hotplate before it has been turned on and before anything has been set on it.

- Connect the power cord and plug it into a working outlet. Plug the thermocouple into the back of the hot plate using the tightly coiled black cord and secure the thermocouple using the provided housing.

- To run a reaction at elevated temperature with precise temperature control, one most often uses an oil bath to heat the flask evenly.  This oil bath usually consists of a glass crystallizing dish partially filled with silicon oil.  Do not add too much silicon oil at first, as the glassware will displace some of the oil and because the oil will expand slightly when heated.  If the oil level is near the lip of the crystallizing dish after the glassware has been submerged in the room temperature oil bath, oil will likely spill out of the dish when it is heated.  A magnetic stir bar or slightly bent metal paper clip should also be placed in the oil bath below the flask to stir the oil and distribute heat evenly.

- The thermocouple should be secured in its housing during use.  Its metal tip should be submerged in the oil without touching the bottom surface of the crystallizing dish.

- Place the apparatus in the oil bath and use a sturdy metal three-prong clamp to secure the apparatus firmly to the metal post on the hot plate or to the metal bars in the fume hood.  Always clamp the bottom-most piece of glassware at its cylindrical neck.  Another clamp can be placed higher if another point of attachment is needed to secure the apparatus, especially if the apparatus is tall, heavy, or if its center of gravity is not directly above the three-prong clamp holding the round-bottomed flask. When the apparatus is submerged in the oil, the level of the oil should be even with or above the solution contained in the glassware.

- When heating is to begin, turn on the Thermocoupled Hot Plate by flipping the on/off switch located on the left hand side of the machine.

- Adjust the speed of the magnetic stirrer by turning the dial on the front right-hand side of the device clockwise.  An ideal stirring speed is as rapid as possible without splashing the liquid or oscillating the magnetic stir bar.  The magnetic stir bar should be placed as close to the surface of the hot plate as possible for ideal stirring control.

- To turn on the heating function of the hot plate, rotate the dial on the front left-hand side of the device clockwise.  Adjust the dial so that the temperature on the digital readout on the front of the device is approximately 30-40 °C higher then the desired heating temperature.  Then use the arrows on the thermocouple to adjust the temperature.  Each time the top arrow is pressed, the set temperature increases by 1 °C.  If the top arrow is held down, the set temperature increases by 1 °C nine times, then increases in increments of 10 °C.

- When the desired temperature is set, close the hood sash to minimize excess cooling of the apparatus caused by increased air flow through the fume hood.  The set temperature will usually reach the desired temperature within a few minutes.

- If leaving the reaction to heat for a prolonged time period, use a Sharpie to write the reaction being performed on the exterior of the hood sash with a complete scheme (reactant  product), notebook number, date, time, and your contact number.  If cooling water and a condenser are being used to heat a reaction mixture overnight at reflux, secure the water tubes to the apparatus using copper wire to reduce the likelihood of a flood. Ensure the water flow is as low as possible (dripping or a very slight stream) to maintain condensation in the condenser.

- When heating is complete and if the reaction is complete, turn both dials counterclockwise until they cannot be turned further and turn off the hot plate.  Raise the apparatus out of the oil bath and allow to cool to ambient temperature.  CAUTION:  the oil will be hot.  Excess oil on the apparatus can be removed with a paper towel moistened with hexane.  Be sure to wear gloves when removing the oil with hexane. If further stirring is desired without heating, turn off the heating only and leave the stirring on.

Approved By (PI) : Stephen Chamberland, Ph.D.					Date: 10/7/14

PI Signature: ___________________________________________________		Date: ____10/7/14____

EH&S Signature: ________________________________________________		Date: ______________