| SOP for: (check one) Process Hazardous Chemical Hazard Class of Chemicals |  |  |
| --- | --- | --- |
| Name of Procedure: Working with Trimethylsilyl Azide (Azidotrimethylsilane) |  |  |
| Applies to Lab (name): Chamberland Research Group | SCI Room #: 313 |  |
| Prepared By: James Siegenthaler, Stephen Chamberland | Date: 9/4/12 | Revision: 9/4/12 |

# Special Notifications:

Do not use if you are taking blood pressure medicine. Has a flash point of 6°C.

# Hazard Description:

This is a highly flammable liquid that is highly toxic either by inhalation or by skin absorption. It targets the nerves and cardiovascular system. It has been reported that individuals taking high blood pressure medicine noticed a dangerous sudden drop in blood pressure after being exposed. The material can release hydrazoic acid, which is known to cause drops in blood pressure. Hydrazoic acid is also extremely flammable and highly toxic.

# Protective Equipment:

- Lab coat

- Protective wrap-around goggles

- Two pairs of standard chemically resistant nitrile gloves or one pair of heavy-duty nitrile gloves

- Long pants

- Close-toed shoes

# Engineering Controls:

# Use only in a well-ventilated chemical fume hood away from sources of ignition. During the utilization of the material, use double gloves, and ensure that the chemical does not come into contact with skin. Use a blast shield when handling the chemical, and ensure that sources of ignition are far away. Before opening the bottle, discharge any electrostatic charge.  After utilization, rinse lab tools well with a non-halogenated solvent before washing. When the chemical is not in use, store under nitrogen, with electrical tape sealing the cap, in a chemical freezer below 6 °C.  The chemical is stable under recommended storage conditions.

# Special Handling or Storage Requirements:

Avoid any contact with skin and eyes, or inhalation of the vapor or mist.

# Store in the chemical freezer keeping the container tightly closed and sealed with electrical tape and Parafilm. Keep the container upright at all times to avoid spills or leakage. This chemical will hydrolyze easily, so store under inert gas, such as nitrogen. The chemical is stable under recommended storage conditions; however, do not store near acids or strong oxidizing agents.

# Spill Response Procedure:

Ensure adequate ventilation and wear respiratory protection.  Remove all sources of ignition and evacuate personal to a safe location. The vapors are dangerous and can accumulate in low areas forming explosive concentrations; care must be given. To clean spills, contain the spillage and then collect with an electrically protected vacuum cleaner or by a chemically resistant absorption pad, and place in a hazardous waste container for disposal. Clean the lab bench thoroughly with soap and water after the spill has been removed.

# Decontamination/Waste Disposal:

Contact EH&S for information on proper disposal. Remove any contaminated clothing and discard as hazardous waste.  Discard any residual reagent and solvents that have come into contact with the reagent into the organic waste container.

# Special Emergency Procedures:

If spilled on exposed skin, wash thoroughly for at least 15 minutes and consult a physician immediately.  If inhaled, remove person to fresh air and consult a physician immediately. Exposure to the chemical has caused individuals to have bloodshot eyes. Monitor blood pressure carefully of the exposed individual to ensure it does not drop to dangerous levels.

Procedure may be performed after normal working hours (M-F, 8-5)?	Yes    No

Procedure may be left unattended?		Yes     No

# Step by Step Procedure:

This liquid is usually stored under nitrogen in the freezer and transferred under nitrogen using a syringe at room temperature.  Consult SOP “Using a syringe” for proper syringe technique and the SOP “Using the double manifold” to learn how to prepare this reagent for syringe transfer under inert gas.  To use, transfer the storage vessel from the freezer to the fume hood and allow it to warm to room temperature under a positive pressure of nitrogen gas.

Approved By (PI) :  Stephen Chamberland, Ph.D.					Date: 9/4/12

PI Signature: ___________________________________________________		Date: ______________

EH&S Signature: ________________________________________________		Date: ______________